Answer any 3 of Choice    Jamere Golden

3)  Your cellphone is capable of taking 48-megapixel photos (uncompressed). Over time you stored 1,000 of these photos in the cell phone's internal storage. You decided to share these photos with 6 of your friends simultaneously. What data rate is required to complete the transmission in 10 minutes?

Answer:

Step 1: Calculate the size of one photo

A 48-megapixel photo means it contains ( 48 x10^6 ) pixels. Assuming each pixel requires 24 bits (3 bytes) to represent RGB (red, green, and blue) values, the size of one photo in bytes is:

Size of one photo = 48 x 10^6 x 3 = 144 x 10^6 bytes = 144 MB

Step 2: Calculate the total size of 1,000 photos

Since you have 1,000 photos:

Total size = 1,000 x 144 MB = 144,000 MB = 144 GB

Step 3: Convert the transmission time to seconds

You want to complete the transmission in 10 minutes, which is:

10 minutes = 10 x 60 = 600 seconds

Step 4: Calculate the required data rate for 1 person

To send 144 GB of data to one person in 600 seconds, the required data rate is:

Data rate for 1 person = 144 GB / 600 seconds = 0.24 GB/s = 240 MB/s

Step 5: Calculate the total data rate for 6 people

Since you’re sending the data to 6 friends simultaneously, the total required data rate becomes:

Total data rate = 6 x 240 MB/s = 1440 MB/s = 1.44  GB/s

Final Answer:

The required data rate to share the 1,000 photos with 6 friends simultaneously in 10 minutes is 1.44 GB/s

4. The bandwidth of a communications channel is 1 MHz. Due to parasitic effects, the channel is affected by a noise level of 100 nW. What should be the power in mW of a signal that travels through the communications channel so that the channel can reach a capacity of 10 Mbps?

C = B log_2(1 + S/N)

Where:

- ( C ) is the channel capacity in bits per second (bps).

- ( B ) is the bandwidth of the channel in Hz.

-( S ) is the signal power in watts (W).

- ( N ) is the noise power in watts (W).

- (S/N) is the signal-to-noise ratio (SNR).

Given:

C=10Mbps=10×10^6bps

B=1 MHz=1×10^6 Hz

N=100 nW=100×10^−9 W=10^−7 W

Need to find ( S ), the signal power in milliwatts (mW).

Step 1: Rearrange Shannon's Formula to solve for S

First, rearrange the capacity formula to isolate (S/N):

C = Blog_2 (1 + S/N)

C/B = \log_2(1 + S/N)

Convert the logarithm to base 10

C/B =log_10(1 +S/N) / (log_10(2))

log_10(1 + S/N) = C/B log_10(2)

Step 2: Substitute known values

Known:

log_10(2) is approx 0.3010

C/B = 10 x 10^6 / 1 x 10^6  = 10

Substituting these into the equation:

log_10(1 + S/N) = 10 x 0.3010 = 3.010

Step 3: Solve for S/N

Now convert the logarithm back to exponential form:

1 + S/N = 10^3.010

1+ S/N is approx 1024.6

S/N is approx 1024.6 - 1 = 1023.6

Step 4: Solve for signal power ( S )

Now solve for ( S ):

S = 1023.6 x N

Substitute   N = 10^-7W:

S = 1023.6 x 10^-7 W = 1.0236 x 10^-4W

Convert ( S ) to milliwatts (1 W = 1000 mW):

S = 1.0236  x 10^-4W x 1000 = 0.10236 mW

Final Answer:

The required signal power should be approximately 0.102 mW for the channel to reach a capacity of 10 Mbps.

5. A 30 mW signal travels over three links. The first link causes the signal to lose a fifth of its power. The second link amplifies the signal by a rate equivalent to 6 dB. The third link causes the signal to lose 60% of its power. What is the power of the signal in mW at the end of the third link?

Given:

- Initial signal power ( P_0 = 30mW)

- First link: loses 1/5 of its power

- Second link: amplifies by 6 dB

- Third link: loses 60% of its power

Step 1: Signal power after the first link

The first link causes the signal to lose ⅕ of its power, meaning 4/5 of the original power remains.

P_1 =(4/5) x 30 mW = 24 mW

Step 2: Signal power after the second link (6 dB amplification)

Amplification by 6 dB corresponds to a multiplication of power by a factor of 10^6/10 = 10^0.6.

P_2 = P_1 x 10^0.6 = 24 x 10^0.6 is approx 24 x 3.981 = 95.54 mW

Step 3: Signal power after the third link

The third link causes the signal to lose 60% of its power, meaning 40% of the power remains.

P_3 = (0.4) x 95.54 mW = 38.216 mW

Final Answer:

The power of the signal at the end of the third link is approximately 38.22 mW.

6. You are asked to design a cluster of 64 high speed computers by connecting them to each other directly. Half of the computers have network cards with 8 ports (a maximum of 8 connections are allowed per computer) while the rest have network cards with 4 ports (a maximum of 4 connections are allowed per computer). Compute the number of network cables needed.

Given:

- Total computers: 64

- Half of the computers have 8 ports: 64/2 = 32 computers

- Half of the computers have 4 ports: 64/2 = 32 computers

Step 1: Maximum connections

1. Computers with 8 ports:

- Each can connect to a maximum of 8 other computers.

- Total connections possible from these computers:

32 x 8 = 256

2.  Computers with 4 ports:

- Each can connect to a maximum of 4 other computers.

- Total connections possible from these computers:

32 x 4 = 128

Step 2: Total connections

The total connections possible from both types of computers is:

256 + 128 = 384

Step 3: Calculating the number of cables

Since each connection between two computers requires one cable, and each connection is counted twice (once for each computer), we divide the total connections by 2 to get the actual number of cables needed:

Number of cables = 384/2 = 192

Final Answer:

The number of network cables needed to connect the 64 computers is 192.