Jamere Golden  Hmw 1

Problem 1

Your organization has four offices networked by four completely connected meshes. The first completely connected mesh interconnects 10 computers, the second 8 computers, the third 5 computers, and the fourth 4 computers. You are asked to integrate the networks into a single completely connected mesh. Determine the number of interconnections that need to be added. Show your work in detail.

Given:

- First network: 10 computers

- Second network: 8 computers

- Third network: 5 computers

- Fourth network: 4 computers

Total computers:

10 + 8 + 5 + 4 = 27  computers.

Total interconnections for 27 computers:

27(27-1)/2 = 27 * 26/ 2 = 351  interconnections.

Existing interconnections:

- First network: 10(10-1)/2 = 45

- Second network: 8(8-1)/2 = 28

- Third network: 5(5-1)/2 = 10

- Fourth network: 4(4-1)/2 = 6

Total existing interconnections:

45 + 28 + 10 + 6 = 89.

Interconnections to be added:

351 - 89 = 262.

Final Answer:

262 interconnections need to be added.

Problem 2

Using Octave, design a composite digital signal using a minimum of 8000 samples. The constituent sinusoids are sines having amplitudes of 6, 2, and 5, respectively. Their respective frequencies are 0.1, 0.15, and 0.25. Plot the composite signal in the frequency domain. Explain in detail the process to reconstitute the original signal from the plot produced in the frequency domain.

n = [0:7999];       % Define the range of samples (8000 samples)

A = [6, 2, 5];               % Amplitudes

F = [0.1, 0.15, 0.25];     % Frequencies

% Define the individual sinusoidal signals

S1 = A(1) * sin(2 * pi * F(1) * n);  % First sine wave

S2 = A(2) * sin(2 * pi * F(2) * n);  % Second sine wave

S3 = A(3) * sin(2 * pi * F(3) * n);  % Third sine wave

% Sum the sinusoidal signals to form the composite signal

composite_signal = S1 + S2 + S3;

% Plot the composite signal in the frequency domain using FFT

figure;

plot(abs(fft(composite_signal)));

title('Composite Signal in Frequency Domain');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

Reconstruction Process

1. Sample Size:  n = [0:7999] , with 8000 samples.

2. Frequency Domain Analysis:

- Peaks in the frequency domain plot indicate the frequencies and amplitudes of the original sinusoids.

3. Identified Frequencies and Amplitudes:

- Frequency Peaks: Approx. 0.1, 0.15, 0.25 Hz.

- Amplitude Peaks: Heights corresponding to 6, 2, 5.

4. Reconstructing the Signal:

- For each frequency component:

- First Component: (6 * sin(2pi * 0.1 * n / 8000)

- Second Component: (2 * sin(2pi * 0.15 * n / 8000)

- Third Component: (5 * sin(2pi * 0.25 * n / 8000)

5. Composite Signal:

Composite Signal= (6 * sin(2pi * 0.1 * n / 8000) + ( 2 * sin(2pi * 0.15 * n / 8000) + (5 * sin(2pi * 0.25 * n / 8000)

This process reconstructs the original signal by combining the identified sinusoidal components.

Problem 3

Write Octave code to play the first four notes of Beethoven’s 5 symphony. Find a way to have small duration pauses between notes when needed. Adjust the duration of the fourth note to last longer as in the symphony. Explain your work.

% Frequencies of the notes

G = 783.99;    % G4

Eb = 622.25;   % Eb5

D = 587.33;    % D5

C = 523.25;    % C5

% Sampling frequency

fs = 2000;

% Duration of each note (in seconds)

durationNote = 0.4;  % Short duration for the first three notes

durationLongNote = 0.8;  % Longer duration for the fourth note

pauseDuration = 0.1; % Duration of pauses between notes

% Time vectors

timeNotes = 0:1/fs:durationNote;

timeLongNote = 0:1/fs:durationLongNote;

timePause = 0:1/fs:pauseDuration;

% Create each note

Gsound = sin(2 * pi * G * timeNotes);

Ebsound = sin(2 * pi * Eb * timeNotes);

Dsound = sin(2 * pi * D * timeNotes);

Csound = sin(2 * pi * C * timeLongNote); % Longer duration

% Create pause

pauseTime = zeros(1, length(timePause));

% Concatenate notes with pauses

composition = [Gsound, pauseTime, Gsound, pauseTime, Gsound, pauseTime, Ebsound, pauseTime, Dsound, pauseTime, Csound];

% Play the sound

sound(composition, fs);

To play the first four notes of Beethoven's 5th Symphony, start by setting the frequencies for the notes G, Eb , D, and C. Use a sampling frequency of 2000 Hz to ensure good audio quality. Set the duration for the first three notes to 0.4 seconds each, and extend the duration of the fourth note (C5) to 0.8 seconds to match the symphony's timing. Incorporate a 0.1-second pause between each note. Generate time vectors for each note and pause, and create sine waves corresponding to the frequencies of the notes. Combine these notes and pauses into a single array. Finally, use the `sound` function to play the resulting composite signal.