CS-440 Data Communications

Homework 2

Problems

- A signal travels over three cables. The signal loses 1/4 of its power in the first cable. At the end of the first cable an amplifier with a gain of 3 dB is used. The second cable causes the signal to lose 20% of its power. At the end of the second cable another amplifier with a gain of 2 dB is used. Finally, the signal loses 10% of its power in the third cable. Determine the overall loss of signal power both as a percentage and in dB. Show all steps followed in detail.

- A connection uses four cables of 1, 2, 3, and 4 km, respectively. The first cable has a loss of 1 dB/km. Assuming that the second cable has 10% more loss per km than the first cable, the third cable has 10% more loss per km than the second cable, and the fourth cable has 10% more loss per km than the third cable what percent of the signal power is lost when it travels through all four cables?

- You want to use a channel to transmit the recorded music of a quartet (four musical instruments). Each instrument will require the recording of 200 different sounds.  If the bandwidth of the channel is 1MHz, what is the theoretical maximum data rate?

- A signal with a power of 1.5 mW travels through a communications line that is noisy at a level of 0.1 mW. The bandwidth of the line is 4 MHz. What is the maximum data rate supported by the line?

Note: Submit your answers as a single compressed zip file to canvas. The file may contain computerized drawings, code, and text as needed.