Jamere Golden

- A signal travels over three cables. The signal loses 1/4 of its power in the first cable. At the end of the

First, an amplifier with a gain of 3 dB is used. The second cable causes the signal to lose 20% of

its power. At the end of the second cable another amplifier with a gain of 2 dB is used. Finally, the

signal loses 10% of its power in the third cable. Determine the overall loss of signal power both as a

percentage and in dB. Show all steps followed in detail.

Let’s denote the initial power of the signal as P_0

Step 1: Power Loss in the First Cable

The first cable causes a loss of  1/4 of the power, which means 3/4 of the power remains.

P_1 = P_0 x (1 - 1/4 = P_0 x 3/4

Step 2: Amplifier Gain After the First Cable

The amplifier adds 3 dB of gain. The power gain in terms of ratio can be calculated using the formula:

Power Gain (ratio) = 10^Gain (dB)/10 = 10^3/10 is approx 2.000

So, the power after the amplifier is:

P_2 = P_1 x  2.000 = (P_0 x 3/4) x 2.000 = P_0 x 3/2

Step 3: Power Loss in the Second Cable

The second cable causes a loss of 20% of the power, meaning 80% remains.

P_3 = P_2 x (1 - 0.20) = P_2 x 0.80

Substituting  P_2 :

P_3 = (P_0 x 3/2) x 0.80 = P_0 x 3 x 0.80/2  = P_0 x 2.4/2 = P_0 x 1.2

Step 4: Amplifier Gain After the Second Cable

The amplifier adds 2 dB of gain.

Power Gain (ratio) = 10^2/10 is  approx 1.585

The power after the amplifier is:

P_4 = P_3 x 1.585 = (P_0 x 1.2) x 1.585 = P_0 x 1.902

Step 5: Power Loss in the Third Cable

The third cable causes a loss of 10%, meaning 90% remains.

P_5 = P_4 x (1 - 0.10) = P_4 x 0.90

Substituting P_4 :

P_5 = (P_0 x 1.902) x 0.90 = P_0 x 1.902 x 0.90 = P_0 x 1.7118

Step 6: Overall Loss in Percentage and in dB

To find the overall loss, we compare the final power  P_5  to the initial power P_0 :

Power Remaining Percentage

The overall power remaining is:

Overall Remaining Percentage = P_5 /P_0 x 100 = P_0 x 1.7118/P_0 x 100 = 171.18%

Overall Loss Percentage

The overall loss in percentage is:

Overall Loss Percentage = 100% - 171.18% = -71.18%

Overall Loss in dB

The overall loss in dB can be calculated using the formula:

Overall Loss (dB)} = 10 log_10(P_5/P_0) = 10 log_10(1.7118)

Calculating this gives:

Overall Loss (dB) is approx 10 x 0.233 = 2.33 dB

Final Results

- Overall remaining power: 171.18%

- Overall loss in percentage:  -71.18%

- Overall loss in dB:  2.33 dB

This indicates that the signal has gained power overall due to the amplifiers exceeding the losses from the cables.

2)  A connection uses four cables of 1, 2, 3, and 4 km, respectively. The first cable has a loss of 1 dB/km.

Assuming that the second cable has 10% more loss per km than the first cable, the third cable has 10%

more loss per km than the second cable, and the fourth cable has 10% more loss per km than the third

cable what percent of the signal power is lost when it travels through all four cables?

Step 1: Loss Calculation for Each Cable

First Cable

- Length: 1 km

- Loss: 1 dB/km

Total Loss (Cable 1) = 1 dB/km x 1 km = 1 dB

Second Cable

- Length: 2 km

- Loss per km: 1 dB/km + 10% = 1 dB/km + 0.1 dB/km = 1.1 dB/km

Total Loss (Cable 2) = 1.1 dB/km x  2 km = 2.2 dB

Third Cable

- Length: 3 km

- Loss per km: 1.1 dB/km + 10% = 1.1 dB/km + 0.11 dB/km = 1.21 dB/km

Total Loss (Cable 3)= 1.21 dB/km x 3km = 3.63 dB

Fourth Cable

- Length: 4 km

- Loss per km: 1.21 dB/km + 10% = 1.21 dB/km + 0.121 dB/km = 1.331 dB/km

Total Loss (Cable 4) = 1.331 dB/km x 4 km = 5.324 dB

Step 2: Total Loss Calculation

Now we sum the losses from all cables:

Total Loss = Total Loss (Cable 1) + Total Loss (Cable 2) + Total Loss (Cable 3) + Total Loss (Cable 4)

Total Loss = 1 dB + 2.2 dB + 3.63 dB + 5.324 dB

Total Loss is approx 12.154 dB

Step 3: Remaining Power Calculation

To find the remaining power, we can use the formula:

Remaining Power Ratio = 10^-Total Loss (dB) / 10

Substituting the total loss:

Remaining Power Ratio = 10^-12.154 / 10  is approx 10^-1.2154 is approx 0.061

Step 4: Percentage of Power Lost

The percentage of power lost is calculated by:

Power Lost Percentage = (1 - Remaining Power Ratio}) x 100%

Power Lost Percentage= (1 - 0.061) x 100% is approx 93.9%

Final Result

Approximately 93.9% The signal power is lost when it travels through all four cables.

3. You want to use a channel to transmit the recorded music of a quartet (four musical instruments).

Each instrument will require the recording of 200 different sounds. If the bandwidth of the channel

is 1MHz, what is the theoretical maximum data rate?

Maximum Data Rate = 2 x B x log_2(L)

Where:

- B is the bandwidth of the channel in hertz,

- L is the number of distinct signal levels (or different sounds).

Given Information

- Bandwidth  B = 1 MHz = 1 x 10^6 Hz

- Each instrument requires the recording of 200 different sounds. Since there are four instruments in total, the total number of different sounds L can be calculated as follows:

L = 200 sounds per instrument = 200

Calculate the Maximum Data Rate

1. Calculate log_2(L):

log_2(200) is approx 7.6439

2. Apply the values to the Nyquist formula:

Maximum Data Rate = 2 x 1 x 10^6 x 7.6439

Maximum Data Rate  is approx 2 x 1,000,000 x 7.6439 is approx 15,287,800 bps

Conclusion

The theoretical maximum data rate for transmitting the recorded music of a quartet over a 1 MHz channel is approximately 15.29 Mbps.

4. A signal with a power of 1.5 mW travels through a communications line that is noisy at a level of 0.1

mW. The bandwidth of the line is 4 MHz. What is the maximum data rate supported by the line?

C = Blog_2(1 + P/N)

Where:

- C is the maximum channel capacity (data rate in bits per second),

- B is the bandwidth of the channel (in hertz),

- P is the power of the signal (in watts),

- N is the power of the noise (in watts).

Given Information

- Signal power P = 1.5 mW = 1.5 x 10^-3W

- Noise power N = 0.1mW = 0.1 x 10^-3W

- Bandwidth B = 4 MHz = 4 x 10^6 Hz

Step 1: Calculate the Signal-to-Noise Ratio (SNR)

SNR = P/N = 1.5 x 10^-3 / 0.1 x 10^-3 = 1.5/0.1 = 15

Step 2: Calculate the Maximum Data Rate C

Now substitute the values into the Shannon-Hartley theorem:

C = Blog_2(1 + SNR)

C = 4 x 10^6log_2(1 + 15)

Calculate (1 + 15)

C = 4 x 10^6log_2(16)

Since 16 is 2^4, we have:

log_2(16) = 4

Now substitute this back into the equation:

C = 4 x 10^6 x 4 = 16 x 10^6 bps

Conclusion

The maximum data rate supported by the line is 16 Mbps.